# MRPL Mangalore Refinery — Crude & Vacuum Distillation Unit (CDU/VDU)

# Standard Operating Procedure: Crude Blend Optimization & Tower Operations

## 1. Feed Optimization & Desalting

The Crude Distillation Unit processes crude blends ranging from sweet light crudes (Arab Light, Bonny Light) to heavy sour crudes (Maya, Basrah Heavy). Electrostatic Desalter D-101 maintains salt content below 3.0 PTB and water cut below 0.1% vol to prevent overhead condenser fouling and naphthenic acid corrosion.

## 2. Column Operating Conditions & Draw Temperatures

| Distillation Cut | Column Tray / Section | Draw Temperature | Quality Specification |
| --- | --- | --- | --- |
| Heavy Naphtha | Tray 38-42 | 140°C – 165°C | RVP < 0.85 bar, N+2A > 55% |
| Aviation Turbine Fuel (ATF / Kerosene) | Tray 28-32 | 195°C – 230°C | Smoke Point > 21 mm, Flash > 38°C |
| High Speed Diesel (HSD / Gas Oil) | Tray 14-18 | 290°C – 345°C | Cetane Index > 48, Sulfur < 10 ppm (BS-VI) |
| Atmospheric Residue (AR) | Column Sump | 345°C – 360°C | Feed to Vacuum Distillation Unit (VDU) |

## 3. Furnace F-101 Coil Protection

Crude charge furnace F-101 skin temperatures must not exceed 480°C. Flow meters FT-201 through FT-204 monitor pass balance across the four radiant coils to prevent localized coke deposition.