# MRPL Mangalore Refinery — Environmental Management Division

# Statutory Compliance Matrix: Karnataka State Pollution Control Board (KSPCB) Standards

## 1. Continuous Online Ambient & Flue Gas Emission Limits

| Emission Source | Pollutant | KSPCB Statutory Limit | MRPL Actual Recorded Average (2025–2026) |
| --- | --- | --- | --- |
| Sulfur Recovery Units (SRU-1/2/3) | Sulfur Dioxide (SO2) | < 250 mg/Nm3 | 142 mg/Nm3 (99.9% sulfur recovery via TGTU) |
| FCCU Regenerator Stack | Particulate Matter (PM) | < 50 mg/Nm3 | 31 mg/Nm3 (Treated via Electrostatic Precipitator ESP-401) |
| Crude Furnaces F-101 / F-201 | Oxides of Nitrogen (NOx) | < 250 mg/Nm3 | 168 mg/Nm3 (Ultra-low NOx burners) |
| Flare Stacks (South & North) | Carbon Monoxide (CO) | < 100 mg/Nm3 | 24 mg/Nm3 (Continuous steam-assisted combustion) |

## 2. Treated Effluent Quality Limits (Discharge to Arabian Sea)

Treated effluent from Effluent Treatment Plants (ETP-1 and ETP-2) must comply with the following limits:
• Phenolic Compounds: < 0.35 mg/L
• Sulfides: < 0.50 mg/L
• Oil and Grease: < 5.0 mg/L
• Chemical Oxygen Demand (COD): < 125 mg/L
• Biochemical Oxygen Demand (BOD 3 days at 27°C): < 15.0 mg/L.