# MRPL Mangalore Refinery — Single Point Mooring (SPM) Operations

# Standard Operating Procedure: Offshore VLCC Crude Tanker Unloading (SOP-SPM-2026-02)

## 1. SPM Facility Description

MRPL operates a Single Point Mooring (SPM) terminal located approximately 16.5 kilometers offshore New Mangalore Port at a water depth of 32 meters. The terminal can handle Very Large Crude Carriers (VLCCs) up to 300,000 DWT. Crude oil is pumped from the tanker through two 24-inch floating hoses, the SPM swivel, a pipeline end manifold (PLEM), and a 48-inch subsea pipeline to the onshore coastal crude oil terminal (CCOT) at Kasaba and onward to the refinery.

## 2. Discharge Operations & Pressure Surge Limits

| Parameter / Safety System | Design Specification | Operating Safety Rule |
| --- | --- | --- |
| Maximum Discharge Rate | 10,000 m3/hour | Initial rate capped at 2,500 m3/h until pipeline pressure stabilizes |
| Subsea Pipeline Pressure Relief Valve PRV-901 | Set point: 18.5 bar(g) | Trips flow and diverts surge to onshore surge relief tank at CCOT |
| Marine Breakaway Coupling (MBC) | Axial load trigger: 450 kN | Guarantees dry break without marine oil spill in extreme tanker drift |